UCBM internship Pre-reading summary

Paper 1: Hybrid robotic systems for upper limb rehabilitation after stroke: A

Review (2016)

- Functional Electrical Stimulation (FES): Low power electrical pulses for muscle contraction

- Hybrid Robotic Rehabilitation (HRR) systems: FES + Robotic devices

- Why HRR?

  - Strokes affect voluntary movement control

  - motor impairment continues even after physio

  - upper arm paralysis – rarely regain functionality for activities of daily life (ADL)

  - FES can induce changes in cortical excitability, thus retraining neurons.

  - (Cortical excitability: The responsiveness of cerebral cortex neurons to a given input, indicating how easily they fire action potentials.)

  - Thus, need for HRR

- Challenges of FES:

  - human body muscles complex and non linear and can’t reliably control all movts with FES

  - Non physiological movt  muscle fatigue

- Robotic devices for:

  - Automates intensive rehabilitation

  - Provides kinematic (position, vel, acc) and kinetic (force, mass, energy) data to track patient evolution + recovery

  - Can use with VR to increase patient engagement

- Assisted-as-needed (ANN) method:

  - Issue: Slacking effect – patients get lazy and make the robot do all the work

  - Leads to no recovery

  - Solution: ANN – robot assistance only when users cannot perform tasks

Review of 14 papers related to upper limb hybrid robotic exoskeleton and FES (grasping, reaching and both)

Technical overview

- Grasping

  - NESS hand master – first system: 5 electrodes for open and close of hand, open loop control and constant pulses

  - Hybrid assistive neuromuscular dynamic stimulation (HANDS): splint with single channel FES, intensity using PWM

  - FES for wrist flexion (up down): assistive parts driven by EMG signals (electric signals from muscles)

Paper 2: Functional Electrical Stimulation (2008)

- Short electrical pulses to artificially induce current in motor neurons, causing paralyzed muscles to contract

- Challenges:

  - muscles fatigue much faster than natural movement since FES:

    - recruits motor units synchronously

    - targets fast-twitch fibers first

  - Muscle response to electrical stimulation is non linear and time varying

  - Must account for involuntary muscle spasms and delays.

  - Closed loop control strategies are much more complex to implement.

Paper 3: Adaptive hybrid robotic system for rehabilitation of reaching movement after a

brain injury: a usability study (2017)

- Hybrid system consists of:

  - Passive exoskeleton (Armeo Spring) - supports the weight of the arm against gravity

  - FES - activates the triceps and anterior deltoid to perform reaching tasks

- Controller: Feedback error learning (FEL)

  - Artificial neural network to learn inverse dynamics of arm as feedforward

  - PID based feedback controller

- Good results, patient reported satisfaction, 62.5% to 83% accuracy over two sessions

Questions:

- I didn’t understand most of the muscle physiology related terms… will need to research on that more. (Research done)

- In paper 1, systems like GO-SAIL assume the movements of the shoulder and elbow are decoupled from the hand and control separately. Biomechanically, how valid is this assumption during 3D reaching, and does this introduce unmodeled dynamic coupling?

- In Dynamics, anything unpredictable and usually stochastic in the state is modelled as noise, how is biological disturbances and non-ideal responses modelled most effectively?

- Using a neural network (Paper 3) is a good idea, but do the weights update fast enough for tracking the rapidly changing non linear force – pulse relation?